AEXNDIS Information File (AEXNDIS.DOC)

--------------------------------------------------

(C) Copyright 1992 Artisoft, Inc.

Edition 1.0

Introduction

============

AEXNDIS.DOS is Artisoft's DOS Network Driver Interface Specification

(NDIS) driver.  AEXNDIS.OS2 is Artisoft's NDIS driver for computers

running OS2.  These Media Access Control (MAC) layer drivers allow you to

use Artisoft Ethernet adapters with network operating systems and

applications that support NDIS.  These drivers can be used with any

Artisoft Ethernet adapter except Artisoft Central Station.

A Word About NDIS

=================

The NDIS protocol, established by Microsoft(R), uses a MAC layer driver

to communicate with the adapter and a protocol level driver to provide

reliable communications between network nodes.  These two layers make up

the transport layer.  A protocol manager program such as Microsoft's

PROTMAN.DOS allows communications between these two programs and also

between any other protocol drivers that need to communicate with the MAC

layer driver.  Once this is done, the computer can communicate

with other network computers.

Software settings for the drivers are stored in the PROTOCOL.INI file.

Here any optional parameters for the drivers can be specified.  For more

information, refer to your network operating system manuals.

Typical network operating systems that support NDIS include:

Microsoft LAN Manager(Tm)

Banyan(R) VINES(Tm)

IBM(R) PC LAN Server(Tm) and Requestor(Tm)

AEXNDIS Installation in a LAN Manager Network

=============================================

Complete the following steps:

1. Install the adapter in your PC as instructed in your hardware manual.

2. Begin the LAN Manager installation by running SETUP.  Refer to your

LAN Manager manuals for more information.

3. When prompted to select a network adapter, select Other.

4. When prompted, insert the Artisoft driver diskette and specify the

drive where you placed the diskette.

5. Proceed with the installation as instructed by SETUP and your user

manuals.

AEXNDIS General Installation Steps

==================================

Complete the following steps to install the AEXNDIS driver if your network

operating system does not have an automated software installation

procedure such as LAN Manager's.

1. Install the adapter in your PC as instructed in your hardware manual.

2. Install your network operating software.  Refer to your network

operating system documentation for more information.

3. Insert the Artisoft driver diskette and copy the software to the

drive where you installed the network operating system software.  Copy

AEXNDIS.DOS if your computer is running DOS.  Copy AEXNDIS.OS2 if your

machine is running OS/2.

4. Edit your CONFIG.SYS to include the commands loading the protocol

manager and AEXNDIS driver.  Below is a sample CONFIG.SYS:

FILES=50

BUFFERS=32

LASTDRIVE=Z

FCBS=16,8

DEVICE=C:\NETWORK\PROTMAN.DOS /i:C:\NETWORK

DEVICE=C:\NETWORK\AEXNDIS.DOS

where "PROTMAN.DOS" refers to the protocol manager provided by your

network operating system.

5. Edit the AUTOEXEC.BAT to include commands loading the rest of the

network TSRs.  For example:

@echo off

prompt $p$g

PATH C:\:C:\WINDOWS;C:\DOS;C:\UTIL;

LOAD NETBEUI

where "NETBEUI" refers to the protocol driver programs for your

network operating system.

6. In your installation directory, create a PROTOCOL.INI file to store the

settings for the protocol manager, protocol driver and the AEXNDIS

driver.  Here is a sample PROTOCOL.INI:

[PROTMAN_XIF]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY = NETBEUI

[NETBEUI_NIF]

Drivername = NETBEUI$

SESSIONS = 6

NCBS = 12

BINDINGS = "AEXNDIS_NIF"

LANABASE = 0

[AEXNDIS_NIF]

IOBASE = 0x300

INTERRUPT = 3

DRIVERNAME = AEXNDS$

Refer to your network operating system software documentation for

information on the valid protocol manager and protocol driver

parameters.

7. Refer to your network operating system manuals for information on using

your network.